| Master Batch No: 366,367,368 |  |  |  |
| --- | --- | --- | --- |
| SECTION–0: MASTER BATCH IDENTIFICATION (ONE TIME) |  |  |  |
| Date of Batch Start | 5/3/26 |  |  |
| No. of Rooms Covered | 4 |  |  |
| Raw Material Basis except minerals | Bagasse old%: Bagasse new%: 47 | Paddy new%: 53 | Mustard old%: Mustard new%: |
| Target Compost Qty (t) | 150MT (50MT each room) |  |  |
| Supervisor / In-charge | RAMARAO |  |  |

| Section 1A: RAW MATERIALS COMPOSITION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Raw materials | Age (months) | Dry(wt) Qty (MT) | Fresh (wt) Qty (MT) | PH | Moisture % | N% | Ash% |
| Paddy straw New | 1 | 28.5 | 33.45 | 6.78 | 14.8 | 0.77 | 12.3 |
| Bagasse new | 3 | 25.5 | 52.5 | 4.18 | 51.4 | 0.35 | 1.6 |
| Chicken Manure | - | 22.5 | 28.02 | 7.91 | 19.7 | 3.54 | 27.8 |
| Gypsum | - | 4.5 | 6.32 | 3.51 | 28.8 | - | 58.1 |
| AS | - | 0.40 | 0.40 | 5.29 | - | 20.5 | - |
| Total quantity |  | 81.4 | 120.69 |  |  |  |  |

| LAGOON WATER QUALITY TESTING FOR PADDY(NEW) SOAKING |  |  |  |
| --- | --- | --- | --- |
|  | Before soaking | After Soaking-1 (3hrs) | After Soaking -2 (24 hrs later) |
| Water volume(lts) | 150 KL | 150 KL | 150 KL |
| Colour | Light Brown | Brown | Brown |
| PH | 7.16 | 6.25 | 7.27 |
| EC | 1.34 | 3.36 | 2.40 |
| TDS | 0.84 | 2.150 | 1.536 |

| 1B. Stage–0 Summary |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Stage | MC % | Temperature (45-55°C) |  |  | Smell(sweet/sour) | Deviations(Y/N) |
|  |  | 0 hrs | 24 hrs | 48 hrs |  |  |
| 0A Pre-wetting |  |  |  |  |  |  |
| 0B (Bunker Loading) | 72.2 | 42.07 | 56.97 | 54.20 | Sweet | N |
| 0D R/L (Reloading) | 70.8 | 47.26 | 59.01 | 54.74 | Sweet | N |
| Unloading |  |  |  |  |  |  |

| ### SECTION–2: PHASE–1 – BUNKER-WISE RECORD |  |  |  |
| --- | --- | --- | --- |
| ### (3 batches loading into 2 bunkers) |  |  |  |
| Loading | BUNKER No: 3 | BUNKER No:6 | Remarks |
| Compost Qty (MT) | 150 | 150 |  |
| Start Fill Height (m) | 2.9 | 2.9 |  |
| Settled Height @ 4h / 8h | 2.6/2.2 | 2.6/2.2 |  |
| MC % | 75.4 | 75.4 |  |
| EC | 1.81 | 1.81 |  |
| Time to Reach (68–72°C) | 11 | 12 |  |
| Holding time (hrs) | 17 | 14 |  |
| Ammonia smell Observation time(hrs) | 5 | 6 |  |
| Ammonia smell (strong/ mild/ weak) | Strong | Strong |  |
| Spring Test (Pass / Fail) | Pass | Pass |  |
| Reloading -1 | BUNKER No:5 | BUNKER No:4 |  |
| Reload Height (m) | 2.9 | 2.9 |  |
| MC % after Reload-1 | 74.6 | 74.6 |  |
| EC | 1.74 | 1.74 |  |
| Time to Reach 68–72°C | 16 | 13 |  |
| Holding time (hrs) | 16 | 15 |  |
| Spring Test (Pass / Fail) | Pass | Pass |  |
| Reloading -2 | BUNKER No:3 | BUNKER No:4 |  |
| Reload Height (m) | 2.8 | 2.8 |  |
| MC % after Reload-2 | 75 | 75 |  |
| EC | 1.93 | 1.93 |  |
| Time to Reach 68–72°C | 14 | 18 |  |
| Holding time (hrs) | 20 | 13 |  |
| Ammonia smell (strong/ mild/ weak) | Mild | Mild |  |

| SECTION–3: PHASE–2 – TUNNEL-WISE RECORD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3A. Tunnel Allocation (2 bunkers filled in 3 tunnels) |  |  |  |  |  |
| Reloading Bunker No: | Tunnel No: | Moisture% | EC | Compost Qty (MT) | Fill Height (m) |
| 3&4 | 10 | 74.8 | 1.79 | 70 | 2.2 |
| 3&4 | 8 | 74.8 | 1.79 | 70 | 2.0 |
| 3&4 | 6 | 74.8 | 1.79 | 70 | 1.6 |

| Stage | Temp Range (°C) | Batch No/Tunnel No Time to reach desired T + Time Held (in hrs) |  |  | Min–Max T probes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 366/10 | 367/8 | 368/6 | 366/10 | 367/8 | 368/6 |
| Levelling + Conditioning-1 (18 – 20 hrs) | 46 - 49 | 7+12 | 7+11 | 17+2 | 48.1-49.7 | 48.1-49.2 | 47.9-48.7 |
| Heating-Up (8-12 hrs) | 59 | 8+2 | 9+1 | 11+0 | 58.9-59.9 | 58.1-59.8 | 57.6-58.9 |
| Pasteurization (8 – 10 hrs) | 59 (58-60) | 9 | 11 | 11 | 58.1-59.2 | 58.6-60.01 | 57.9-59.1 |
| Cooling-1 (12 – 14 hrs) | 60-48 | 13+0 | 10+2 | 8+5 | 50.01-53.54 | 45.61-50.55 | 48.69-49.01 |
| Conditioning-2 (75 - 80 hrs) | 48-45 | 111 | 89 | 92 | 44.97-47.73 | 44.92-49.10 | 46.65-47.32 |
| Cooling-2 (8 - 14hrs) | 24 | 14+4 | 7+10 | 9+7 | 23.50-25.39 | 24.45-29.45 | 22.93-23.91 |

| SECTION–4: COMPOST OUT – TUNNEL-WISE QC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tunnel No | Final MC % | N % | pH | EC | Spring | Color (Grey brown/dark chocolate) | Actinomycetes (Visible-weak/ mild/strong) | Smell (Sour/ Earthy) |
| 8 | 68.2 | 1.94 | 7.39 | 1.95 | Pass | Dark chocolate | Strong | Earthy |
| 6 | 69 | 1.96 | 7.49 | 1.86 | Pass | Dark chocolate | Strong | Earthy |
| 10 | 66.8 | 2.03 | 7.29 | 1.85 | Pass | Dark chocolate | Mild | Earthy |

| Sr. No | Hour | 366/10 |  |  | 367/8 |  |  | 368/6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan |
| 1 | 0hrs | 50.93 | 32.13 | 50.51 | 47.2 | 32.01 | 70.6 | 47.98 | 21.63 | 73.49 |
| 2 | 4hrs | 51.49 | 25.11 | 70 | 49.11 | 45.95 | 65.39 | 48.21 | 25.52 | 69.24 |
| 3 | 8hrs | 49.9 | 14.11 | 72.07 | 48.71 | 37.01 | 72.56 | 48.11 | 26.42 | 68.27 |
| 4 | 12hrs | 50.08 | 16.98 | 78.21 | 48.2 | 34.08 | 78.75 | 48.07 | 31.36 | 68.27 |
| 5 | 16hrs | 49.52 | 23.02 | 82.24 | 47.23 | 37.01 | 81.76 | 47.64 | 28.51 | 65.23 |
| 6 | 20hrs | 46.15 | 16.02 | 83.33 | 45.4 | 26.02 | 75.75 | 47.01 | 28.77 | 58.85 |
| 7 | 24hrs | 45.86 | 26.08 | 77.06 | 45.29 | 15.75 | 69.56 | 47.18 | 29.08 | 52.65 |
| 8 | 28hrs | 47.52 | 29.03 | 78.21 | 47.25 | 37.99 | 65.57 | 47.39 | 29.74 | 51.63 |
| 9 | 32hrs | 47.37 | 17 | 84.49 | 46.85 | 30.06 | 70.6 | 47.06 | 25.01 | 51.62 |
| 10 | 36hrs | 46.88 | 6.06 | 87.48 | 46.74 | 23.06 | 70.6 | 46.96 | 21.69 | 54.84 |
| 11 | 40hrs | 46.66 | 31.02 | 87.64 | 46.83 | 37.01 | 71.59 | 46.96 | 33.18 | 55.87 |
| 12 | 44hrs | 45.12 | 0.09 | 82.24 | 44.45 | 0.23 | 66.56 | 45.48 | 23 | 51.71 |
| 13 | 48hrs | 46.2 | 0.09 | 67.87 | 45.81 | 23.06 | 60.31 | 45.81 | 24.75 | 50.59 |
| 14 | 52hrs | 47.84 | 10.11 | 70.06 | 46.4 | 20.99 | 54.36 | 46.08 | 19.54 | 50.59 |
| 15 | 56hrs | 48.36 | 19.08 | 72.06 | 47.5 | 32.01 | 58.34 | 46.33 | 19.7 | 50.59 |
| 16 | 60hrs | 47.53 | 13.07 | 81.2 | 48.06 | 1.59 | 9.63 | 46.54 | 23.42 | 50.59 |
| 17 | 64hrs | 46.98 | 29.04 | 81.2 | 46.72 | 37.99 | 67.35 | 46.22 | 30.61 | 50.59 |
| 18 | 68hrs | 45.65 | 9.01 | 77.07 | 45.22 | 0.23 | 52.15 | 45.78 | 25.78 | 11.47 |
| 19 | 72hrs | 45.97 | 0.09 | 66.89 | 46.69 | 34.08 | 50.12 | 45.52 | 22.86 | 50.59 |
| 20 | 76hrs | 47.29 | 0.09 | 64.87 | 46.41 | 10.85 | 50.12 | 45.91 | 23.02 | 50.59 |
| 21 | 80hrs | 47.73 | 4.09 | 71.03 | 47.41 | 34.08 | 53.11 | 45.96 | 21.03 | 50.59 |

BATCH 366–367–368

1️⃣ RAW MATERIAL COMPOSITION — BIOLOGICAL READING

Structure

- New paddy + new bagasse

- No mustard / no wheat

👉 Verdict
✔ Simple and clean structure
✔ Bagasse provides porosity
⚠ No structural stabilizer → higher airflow sensitivity

Nitrogen

- CM strong

- AS moderate (0.40 t)

- Compost-out N: 1.94–2.03%

👉 Slightly high N compost
✔ Strong biology
⚠ More responsive to CO₂ → pin-density sensitive

2️⃣ LAGOON WATER & SALT LOAD

- EC before soak: 1.34

- After soak-1: 3.36

- After soak-2: 2.40

👉 Verdict
✔ Salt washed well
✔ No salt stress
✔ Clean compost base

3️⃣ STAGE-0 PERFORMANCE

- 0B MC: 72.2%

- Reload MC: 70.8%

- Temp peak: ~59°C

- Smell: Sweet

👉 Interpretation
✔ Moisture good
✔ Clean aerobic start
⚠ Slightly high temperature

This indicates:

- Strong microbial activity

- Slight early acceleration

4️⃣ PHASE-1 (BUNKER) ANALYSIS

Loading

- Time to peak: 11 vs 12 hrs

- Holding: 17 vs 14 hrs

👉 Good alignment

Reload-1

- Peak: 16 vs 13 hrs

- Holding: 16 vs 15 hrs

👉 Well balanced

Reload-2

- Peak: 14 vs 18 hrs

- Holding: 20 vs 13 hrs ❗

👉 Moderate variation appears

🔴 PHASE-1 VERDICT

✔ Generally stable
⚠ Reload-2 imbalance present

👉 Effect:

- Slight variation in compost maturity

- Can create timing differences at pinning

5️⃣ PHASE-2 ANALYSIS

Tunnel Entry

- MC: 74.8%

- EC: 1.79

- Fill heights: 2.2 / 2.0 / 1.6 m

👉 Observations
✔ Entry condition very good
⚠ Fill height variation significant

🔴 KEY PHASE-2 ISSUE

👉 Tunnel 10 = 2.2 m (high)
👉 Tunnel 6 = 1.6 m (low)

This causes:

- different airflow resistance

- different conditioning intensity

- uneven compost respiration

6️⃣ COMPOST-OUT QC

- MC: 66.8–69%

- pH: 7.29–7.49

- EC: 1.85–1.95

- N: 1.94–2.03

- Actinomycetes: Mild–Strong

- Color: Dark chocolate

- Smell: Earthy

✅ FINAL QC VERDICT

✔ Clean compost
✔ Good maturity
✔ Low EC
✔ Strong biology

7️⃣ BIOLOGICAL CHARACTER OF THIS BATCH

Strength

- Clean (low EC)

- Strong nitrogen

- Good moisture

- Good compost-out

Sensitivity

- No mustard → airflow sensitive

- Slight Phase-1 variation

- Tunnel fill variation

- High biological responsiveness

8️⃣ EXPECTED GROWING ROOM BEHAVIOR

If handled correctly:

- Good SR and CR

- Good yield

- Moderate pin density

If mishandled:

| Observation | Cause |
| --- | --- |
| Striping | Air velocity lanes |
| Uneven pin timing | Phase-1 variation |
| Dense pin clusters | Early low CO₂ |
| Small mushrooms | Over-pinning |
| Soil on caps | Surface wet correction |

9️⃣ AIR VELOCITY SENSITIVITY

👉 This batch = HIGHLY AIR-SENSITIVE

Because:

- No mustard

- Strong compost

- Low EC (fast response)

👉 Small airflow differences will become visible as:

- striping

- lane pinning

🔟 FINAL TECHNICAL VERDICT

Batch 366–368 is BIOLOGICALLY STRONG and CLEAN but STRUCTURALLY AIR-SENSITIVE.

✔ Compost quality: Good
✔ Salt stress: None
✔ Phase-1: Mostly stable
⚠ Main risks:

- airflow imbalance

- tunnel fill variation

- aggressive venting

🧠 ONE-LINE FIELD SUMMARY

👉 “366–368 is strong clean compost — performance depends on airflow control; striping, if any, will be ventilation-driven, not compost-driven.”

🎯 MOST IMPORTANT CORRECTIONS

1️⃣ Tunnel filling

- Keep fill height uniform

- Avoid >2.0 m variation

2️⃣ Ventilation

- Avoid deep CO₂ drop (1500 early)

- Use softer band: 2200 ↔ 1600–1700

3️⃣ Air velocity

- Reduce directional airflow

- Avoid strong OA pulses