| Master Batch No:372,373,374 |  |  |  |
| --- | --- | --- | --- |
| SECTION–0: MASTER BATCH IDENTIFICATION (ONE TIME) |  |  |  |
| Date of Batch Start | 11/03/26 |  |  |
| No. of Rooms Covered | 3 |  |  |
| Raw Material Basis except minerals | Bagasse new%:36 | Paddy new%: 53 | Wheat Straw%:11 |
| Target Compost Qty (t) | 150MT (50MT each room) |  |  |
| Supervisor / In-charge | RAMARAO |  |  |

| Section 1A: RAW MATERIALS COMPOSITION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Raw materials | Age (months) | Dry(wt) Qty (MT) | Fresh (wt) Qty (MT) | PH | Moisture % | N% | Ash% |
| Paddy straw new | 1 | 29.4 | 34.0 | 6.73 | 13.6 | 0.77 | 12.7 |
| Bagasse new | 3 | 20.1 | 46.53 | 4.39 | 12.5 | 0.70 | 10.3 |
| Wheat straw | 1 | 6.0 | 6.8 | 5.58 | 56.8 | 0.35 | 2.2 |
| Chicken Manure |  | 21.0 | 26.1 | 7.92 | 19.6 | 3.36 | 29.2 |
| Gypsum |  | 4.35 | 6.1 | 3.67 | 28.7 | - | 84.4 |
| AS |  | 0.80 | 0.80 | 5.36 | - | 20.5 | - |
| Total quantity |  | 81.65 | 120.33 |  |  |  |  |

| LAGOON WATER QUALITY TESTING FOR PADDY(NEW) SOAKING |  |  |  |
| --- | --- | --- | --- |
|  | Before soaking | After Soaking-1 (3hrs) | After Soaking -2 (24 hrs later) |
| Water volume(lts) | 150 KL | 150 KL | 150 KL |
| Colour | Light brown | Brown | Brown |
| PH | 6.89 | 6.50 | 7.30 |
| EC | 1.07 | 3.57 | 1.45 |
| TDS | 0.68 | 2.28 | 0.92 |

| 1B. Stage–0 Summary |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Stage | MC % | Temperature (45-55°C) |  |  | Smell(sweet/sour) | Deviations(Y/N) |
|  |  | 0 hrs | 24 hrs | 48 hrs |  |  |
| 0A Pre-wetting | 71.3 |  |  |  |  |  |
| 0B (Bunker Loading) | 72.9 | 48.50 | 58.4 | 56.9 | Sweet | N |
| 0D R/L (Reloading) | 70 | 47.30 | 55.66 | 61.70 | Sweet | N |
| Unloading |  |  |  |  |  |  |

| ### SECTION–2: PHASE–1 – BUNKER-WISE RECORD |  |  |  |
| --- | --- | --- | --- |
| ### (3 batches loading into 2 bunkers) |  |  |  |
| Loading | BUNKER No: 2 | BUNKER No:1 | Remarks |
| Compost Qty (MT) | 150 | 150 |  |
| Start Fill Height (m) | 2.9 | 2.9 |  |
| Settled Height @ 4h / 8h |  |  |  |
| MC % | 75.2 | 75.2 |  |
| EC | 1.98 | 1.98 |  |
| Time to Reach (68–72°C) | 13 | 12 |  |
| Holding time (hrs) | 33 | 30 |  |
| Ammonia smell Observation time(hrs) | 5 | 5 |  |
| Ammonia smell (strong/ mild/ weak) | Strong | Strong |  |
| Spring Test (Pass / Fail) | Pass | Pass |  |
| Reloading -1 | BUNKER No:5 | BUNKER No:6 |  |
| Reload Height (m) | 2.9 | 2.9 |  |
| MC % after Reload-1 | 74.5 | 74.5 |  |
| EC | 1.91 | 1.91 |  |
| Time to Reach 68–72°C | 22 | 12 |  |
| Holding time (hrs) | 6 | 19 |  |
| Spring Test (Pass / Fail) | Pass | Pass |  |
| Reloading -2 | BUNKER No:4 | BUNKER No:5 |  |
| Reload Height (m) | 2.8 | 2.8 |  |
| MC % after Reload-2 | 75.7 | 75.7 |  |
| EC | 1.86 | 1.90 |  |
| Time to Reach 68–72°C | 10 | 20 |  |
| Holding time (hrs) | 34 | 20 |  |
| Ammonia smell (strong/ mild/ weak) | Mild | Mild |  |

| SECTION–3: PHASE–2 – TUNNEL-WISE RECORD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3A. Tunnel Allocation (2 bunkers filled in 3 tunnels) |  |  |  |  |  |
| Reloading Bunker No: | Tunnel No: | Moisture% | EC | Compost Qty (MT) | Fill Height (m) |
| 4&5 | 1 | 74.8 | 1.74 | 70 | 1.75 |
| 4&5 | 9 | 74.8 | 1.74 | 70 | 2.0 |
| 4&5 | 11 | 74.8 | 1.74 | 70 | 2.10 |

| Stage | Temp Range (°C) | Batch No/Tunnel No Time to reach desired T + Time Held (in hrs) |  |  | Min–Max T probes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 372/1 | 373/9 | 374/11 | 372/1 | 373/9 | 374/11 |
| Levelling + Conditioning-1 (18 – 20 hrs) | 46 - 49 | 11+8 | 5+14 | 2+17 | 47.0-51.04 | 47.8-49.64 | 46.25-49.98 |
| Heating-Up (8-12 hrs) | 59 | 11+0 | 10+1 | 10+0 | 57.68-60.01 | 57.24-59.80 | 58.08-59.71 |
| Pasteurization (8 – 10 hrs) | 59 (58-60) | 9 | 9 | 9 | 58.08-60.21 | 57.91-59.99 | 57.05-58.74 |
| Cooling-1 (12 – 14 hrs) | 60-48 | 8+5 | 8+5 | 9+4 | 46.91-48.65 | 45.77-50.68 | 47.55-48.91 |
| Conditioning-2 (75 - 80 hrs) | 48-45 | 93 | 90 | 105 | 46.03-47.96 | 45.27-48.86 | 46.46-48.17 |
| Cooling-2 (8 - 14hrs) | 24 | 25+12 | 13+0 | 26 | 24.88-27.97 | 24.32-25.72 | 24.34-25.18 |

| SECTION–4: COMPOST OUT – TUNNEL-WISE QC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tunnel No | Final MC % | N % | pH | EC | Spring | Color (Grey brown/dark chocolate) | Actinomycetes (Visible-weak/ mild/strong) | Smell (Sour/ Earthy) |
| 9 | 67.1 | 2.01 | 7.37 | 1.74 | Pass | Dark chocolate | Strong | Earthy |
| 1 | 66.5 | 1.86 | 7.33 | 1.74 | Pass | Dark chocolate | Strong | Earthy |
| 11 | 67.2 | 1.89 | 7.39 | 1.69 | Pass | Dark chocolate | Strong | Earthy |

| Sr. No | Hour | 372/1 |  |  | 373/9 |  |  | 374/11 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan |
| 1 | 0hrs | 47.31 | 0.15 | 83.92 | 47.93 | 29.96 | 74.77 | 46 | 18.12 | 87 |
| 2 | 4hrs | 50.75 | 0.14 | 84.98 | 47.92 | 29.97 | 68.42 | 47.48 | 31.04 | 80.85 |
| 3 | 8hrs | 50.06 | 38.33 | 91.23 | 48.33 | 29.97 | 66.52 | 47.78 | 37.19 | 75.63 |
| 4 | 12hrs | 47.84 | 21.75 | 89.17 | 48.36 | 29.96 | 70.5 | 45.97 | 27.08 | 69.66 |
| 5 | 16hrs | 48.4 | 30.04 | 90.15 | 47.51 | 29.96 | 73.8 | 46.73 | 41.01 | 63.39 |
| 6 | 20hrs | 47.63 | 21.72 | 69.41 | 46.69 | 29.97 | 68.49 | 46.63 | 21.07 | 57.19 |
| 7 | 24hrs | 47.29 | 29.02 | 75.69 | 46.21 | 29.97 | 62.31 | 47.65 | 29.07 | 58.23 |
| 8 | 28hrs | 47.39 | 18.64 | 76.72 | 46.48 | 22.02 | 57.3 | 47.67 | 29.07 | 59.22 |
| 9 | 32hrs | 47.69 | 20.47 | 82.97 | 46.52 | 22.02 | 56.32 | 47.47 | 28.08 | 65.42 |
| 10 | 36hrs | 47.69 | 20.47 | 82.98 | 47.62 | 29.96 | 61.34 | 46.88 | 25.99 | 68.58 |
| 11 | 40hrs | 47.42 | 18.68 | 89.17 | 47.66 | 29.96 | 66.53 | 46.26 | 32.03 | 67.64 |
| 12 | 44hrs | 47.19 | 20.49 | 92.5 | 47.13 | 29.97 | 71.54 | 45.09 | 3.1 | 63.4 |
| 13 | 48hrs | 46.84 | 32.8 | 83.94 | 45.2 | 29.97 | 68.48 | 46.48 | 34.11 | 58.23 |
| 14 | 52hrs | 46.17 | 15.54 | 79.94 | 46 | 7.09 | 50.09 | 46.56 | 0.08 | 53.1 |
| 15 | 56hrs | 47.12 | 27.58 | 80.97 | 47.5 | 29.97 | 52.11 | 46.57 | 0.08 | 53.1 |
| 16 | 60hrs | 47.15 | 15.61 | 80.98 | 47.87 | 39.85 | 58.27 | 46.46 | 0.08 | 50 |
| 17 | 64hrs | 47.26 | 17.52 | 87.17 | 47.23 | 39.84 | 64.5 | 46.4 | 0.08 | 50.01 |
| 18 | 68hrs | 47.06 | 21.75 | 92.52 | 46.37 | 39.85 | 65.48 | 46.49 | 43.01 | 50.24 |
| 19 | 72hrs | 46.52 | 19.47 | 81.94 | 44.43 | 16.99 | 59.32 | 45.61 | 0.08 | 50.12 |
| 20 | 76hrs | 46.81 | 26.46 | 79.94 | 45.7 | 0.19 | 39.9 | 46.84 | 27.1 | 50.03 |
| 21 | 80hrs | 46.63 | 11.32 | 75.69 | 47.52 | 36.87 | 41.91 | 47.15 | 31.04 | 54.24 |

🔬 BATCH 372–373–374

1️⃣ RAW MATERIAL COMPOSITION — BIOLOGICAL READING

Structure
New paddy 53%, new bagasse 36%, wheat 11%.

👉 Verdict
✔ Balanced paddy–bagasse–wheat structure
✔ Wheat gives useful fiber stability
⚠ No mustard, so this batch is still moderately sensitive to airflow lanes during pinning

2️⃣ LAGOON WATER & SALT LOAD

EC moved from 1.07 → 3.57 → 1.45.

👉 Verdict
✔ Excellent washing
✔ Very low final salt load
✔ This is not a salt-stress batch

So any patching later should not be blamed on lagoon EC.

3️⃣ STAGE-0 PERFORMANCE

MC was 72.9% at loading and 70% at reload. Smell remained sweet, but reload temperature reached 61.7°C.

👉 Interpretation
✔ Moisture is workable
✔ No anaerobic smell
⚠ Reload temperature is too high

This suggests early heat concentration. It does not mean compost failure, but it increases the chance of non-uniform biological behavior later.

4️⃣ PHASE-1 (BUNKER) ANALYSIS

Loading

- Time to 68–72°C: 13 vs 12 hrs

- Holding: 33 vs 30 hrs

👉 Good alignment.

Reload-1

- Time to peak: 22 vs 12 hrs

- Holding: 6 vs 19 hrs

👉 Major imbalance.

Reload-2

- Time to peak: 10 vs 20 hrs

- Holding: 34 vs 20 hrs

👉 Again major imbalance.

PHASE-1 VERDICT

✔ Biology is strong
❗ Compost conditioning is not uniform

This is the main technical weakness of the batch. The batch carries mixed-strength compost into Phase-2.

5️⃣ PHASE-2 ANALYSIS

Tunnel entry was good:

- MC 74.8%

- EC 1.74

- Fill heights 1.75 / 2.0 / 2.1 m.

Pasteurization was controlled at 9 hrs in all three tunnels, but Conditioning-2 ran 93 / 90 / 105 hrs, and Tunnel 11 was clearly longer than the others.

👉 Interpretation
✔ Tunnel entry condition is good
✔ Pasteurization is stable
⚠ Tunnel 11 is slightly over-conditioned
⚠ Fill-height variation is also present

So Phase-2 is acceptable, but it does not fully remove the Phase-1 variation.

6️⃣ COMPOST-OUT QC

Final QC:

- MC 66.5–67.2%

- pH 7.33–7.39

- EC 1.69–1.74

- Spring Pass

- Color Dark chocolate

- Actinomycetes Strong

- Smell Earthy.

👉 Verdict
✔ Compost-out quality is good
✔ EC is low
✔ Final compost is usable and mature

7️⃣ BIOLOGICAL CHARACTER OF THIS BATCH

Strengths
Low salt load, clean Stage-0 smell, good tunnel entry, strong compost-out.

Weaknesses
Major bunker non-uniformity, high reload temperature, one tunnel slightly over-conditioned.

8️⃣ EXPECTED GROWING-ROOM BEHAVIOR

This batch can still produce acceptable results, but it is more likely to show timing variation than a uniform batch.

| Observation | Likely Cause |
| --- | --- |
| Patchy pinning | Phase-1 imbalance |
| Early/late zones | mixed compost strength |
| Striping | airflow amplifying compost variation |
| Uneven pin density | bunker-to-bunker conditioning difference |

9️⃣ AIR VELOCITY SENSITIVITY

👉 Moderate sensitivity

Because wheat is present, the structure is more stable than plain paddy+bagasse batches. But the real driver here is not airflow alone. The bigger problem is that airflow will amplify an already non-uniform compost.

🔟 FINAL TECHNICAL VERDICT

Batch 372–374 is a biologically strong but non-uniform batch.

✔ Salt stress: not an issue
✔ Compost-out: good
❗ Main weakness: Phase-1 imbalance
⚠ Secondary weakness: fill-height and Phase-2 duration variation

🧠 ONE-LINE FIELD SUMMARY

372–374 is a good compost-quality batch, but patch risk comes mainly from bunker non-uniformity, not from salt or formula.