| Master Batch No: 381,382,383 |  |  |  |
| --- | --- | --- | --- |
| SECTION–0: MASTER BATCH IDENTIFICATION (ONE TIME) |  |  |  |
| Date of Batch Start | 18/03/26 |  |  |
| No. of Rooms Covered | 3 |  |  |
| Raw Material Basis except minerals | Bagasse old%: Bagasse new%: 36.2 | Paddy new%: 53 | Wheat Straw%: 10.8 Mustard new%: |
| Target Compost Qty (t) | 150MT (50MT each room) |  |  |
| Supervisor / In-charge | RAMARAO |  |  |

| Section 1A: RAW MATERIALS COMPOSITION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Raw materials | Age (months) | Dry(wt) Qty (MT) | Fresh (wt) Qty (MT) | PH | Moisture % | N% | Ash% |
| Paddy straw new | 2 | 29.4 | 33.5 | 6.84 | 12.2 | 0.77 | 12.7 |
| Bagasse new | 4 | 20.1 | 47.7 | 5.60 | 57.8 | 0.46 | 2.20 |
| Wheat straw | 1 | 6.0 | 6.81 | 5.56 | 11.9 | 0.70 | 10.3 |
| Chicken Manure |  | 20.1 | 24.25 | 8.09 | 17.1 | 3.42 | 30.6 |
| Gypsum |  | 4.2 | 5.89 | 3.64 | 28.7 | - | 84.5 |
| AS |  | 0.60 | 0.60 | 5.28 | - | 20.5 | - |
| Total quantity |  | 80.4 | 118.75 |  |  |  |  |

| LAGOON WATER QUALITY TESTING FOR PADDY(NEW) SOAKING |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Before soaking | After Soaking-1 | After Soaking -2 | After Soaking – 3 |
| Water volume(lts) | 150 KL | 150 KL | 150 KL | 150 KL |
| Colour | Light Brown | Brown | Brown | Brown |
| PH | 7.32 | 6.27 | 7.76 | 7.53 |
| EC | 1.37 | 3.83 | 2.37 | 1.89 |
| TDS | 1.42 | 6.0 |  | 2.50 |

| 1B. Stage–0 Summary |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Stage | MC % | Temperature (45-55°C) |  |  | Smell(sweet/sour) | Deviations(Y/N) |
|  |  | 0 hrs | 24 hrs | 48 hrs |  |  |
| 0A Pre-wetting |  |  |  |  |  |  |
| 0B (Bunker Loading) | 73.6 | 40.01 | 59.30 | 57.42 | Sweet | Y |
| 0D R/L (Reloading) | 72.5 | 45.41 | 58.96 | 56.99 | Sweet | N |
| Unloading |  |  |  |  |  |  |

| ### SECTION–2: PHASE–1 – BUNKER-WISE RECORD |  |  |  |
| --- | --- | --- | --- |
| ### (3 batches loading into 2 bunkers) |  |  |  |
| Loading | BUNKER No: 5 | BUNKER No: 4 | Remarks |
| Compost Qty (MT) | 150 | 150 |  |
| Start Fill Height (m) | 2.9 | 2.9 |  |
| Settled Height @ 4h / 8h | 2.6/2.2 | 2.6/2.2 |  |
| MC % | 74.9 | 74.9 |  |
| EC | 1.85 | 1.85 |  |
| Time to Reach (68–72°C) | 12 | 5 |  |
| Holding time (hrs) | 30 | 29 |  |
| Ammonia smell Observation time(hrs) | 5 | 6 |  |
| Ammonia smell (strong/ mild/ weak) | Strong | Strong |  |
| Spring Test (Pass / Fail) | Pass | Pass |  |
| Reloading -1 | BUNKER No: 5 | BUNKER No: 6 |  |
| Reload Height (m) | 2.9 | 2.9 |  |
| MC % after Reload-1 | 74.5 | 74.5 |  |
| EC | 1.75 | 1.75 |  |
| Time to Reach 68–72°C | 19 | 15 |  |
| Holding time (hrs) | 22 | 29 |  |
| Spring Test (Pass / Fail) | Pass | Pass |  |
| Reloading -2 | BUNKER No: 7 | BUNKER No: 8 |  |
| Reload Height (m) | 2.8 | 2.8 |  |
| MC % after Reload-2 | 75 | 75 |  |
| EC | 1.61 | 1.61 |  |
| Time to Reach 68–72°C | 11 | 12 |  |
| Holding time (hrs) | 23 | 19 |  |
| Ammonia smell (strong/ mild/ weak) | Mild | Mild |  |

| SECTION–3: PHASE–2 – TUNNEL-WISE RECORD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3A. Tunnel Allocation (2 bunkers filled in 3 tunnels) |  |  |  |  |  |
| Reloading Bunker No: | Tunnel No: | Moisture% | EC | Compost Qty (MT) | Fill Height (m) |
| 7&8 | 10 | 74 | 1.50 | 70 | 2.1 |
| 7&8 | 3 | 74 | 1.50 | 70 | 1.85 |
| 7&8 | 4 | 74 | 1.50 | 70 | 1.95 |

| Stage | Temp Range (°C) | Batch No/Tunnel No Time to reach desired T + Time Held (in hrs) |  |  | Min–Max T probes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 381/10 | 382/3 | 383/4 | 381/10 | 382/3 | 383/4 |
| Levelling + Conditioning-1 (18 – 20 hrs) | 46 - 49 | 3+16 | 3+15 | 7+12 | 47.34-49.82 | 48.02-48.84 | 46.49-51.35 |
| Heating-Up (8-12 hrs) | 59 | 15+4 | 10+1 | 10+1 | 58.0-59.94 | 56.97-58.58 | 56.56-59.97 |
| Pasteurization (8 – 10 hrs) | 59 (58-60) | 11 | 9 | 9 | 56.71-59.88 | 57.25-59.23 | 57.51-59.81 |
| Cooling-1 (12 – 14 hrs) | 60-48 | 10+3 | 8+4 | 9+4 | 46.32-48.97 | 47.84-48.42 | 45.59-49.46 |
| Conditioning-2 (75 - 80 hrs) | 48-45 | 91 | 91 | 100 | 45.09-48.13 | 45.93-46.39 | 44.03-47.68 |
| Cooling-2 (8 - 14hrs) | 24 | 13+0 | 15+0 | 5+16 | 25.10-26.35 | 23.58-25.53 | 22.2-23.32 |

| SECTION–4: COMPOST OUT – TUNNEL-WISE QC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tunnel No | Final MC % | N % | pH | EC | Spring | Color (Grey brown/dark chocolate) | Actinomycetes (Visible-weak/ mild/strong) | Smell (Sour/ Earthy) |
| 3 | 66.3 | 2.08 | 7.64 | 1.73 | Pass | Dark Chocolate | Mild | Earthy |
| 10 | 66.8 | 2.03 | 7.49 | 1.80 | Pass | Dark Chocolate | Strong | Earthy |
| 4 | 66.2 | 1.89 | 7.34 | 2.05 | Pass | Dark Chocolate | Strong | Earthy |

| Sr. No | Hour | 381/10 |  |  | 382/3 |  |  | 383/4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan |
| 1 | 0hrs | 47.7 | 4.09 | 67.86 | 47.99 | 34 | 64.74 | 48.03 | 27.37 | 71.06 |
| 2 | 4hrs | 49.09 | 26.08 | 66.88 | 47.7 | 23.04 | 60.57 | 48.16 | 31.94 | 68.85 |
| 3 | 8hrs | 46.68 | 21.04 | 64.87 | 47.7 | 25.32 | 60.57 | 45.85 | 24.37 | 62.68 |
| 4 | 12hrs | 46.53 | 24.01 | 58.67 | 47.75 | 24.25 | 60.57 | 46.53 | 31 | 57.08 |
| 5 | 16hrs | 46.47 | 4.08 | 52.51 | 47.72 | 34.06 | 62.58 | 46.69 | 27.1 | 51.19 |
| 6 | 20hrs | 48.27 | 30.03 | 53.57 | 46.42 | 26.29 | 60.57 | 47.33 | 31.43 | 53.2 |
| 7 | 24hrs | 48.63 | 16.02 | 57.69 | 45.43 | 21.28 | 60.57 | 47.53 | 30.98 | 58.4 |
| 8 | 28hrs | 47.73 | 14.05 | 62.68 | 46.02 | 21.25 | 60.57 | 45.97 | 31.34 | 60.42 |
| 9 | 32hrs | 45.83 | 17.21 | 63.91 | 46.22 | 21.26 | 60.57 | 45.25 | 21.69 | 54.42 |
| 10 | 36hrs | 46.25 | 12.05 | 50.51 | 46.62 | 25.33 | 63.6 | 45.84 | 31.31 | 52.6 |
| 11 | 40hrs | 46.61 | 8.02 | 50.5 | 46.46 | 34.11 | 60.57 | 46.23 | 25.3 | 50.2 |
| 12 | 44hrs | 47.13 | 7.04 | 50.5 | 44.63 | 4.81 | 50.35 | 46.66 | 27.35 | 50.2 |
| 13 | 48hrs | 47.88 | 19.08 | 57.69 | 44.92 | 0.04 | 58.56 | 46.77 | 24.8 | 50.2 |
| 14 | 52hrs | 47.46 | 29.04 | 62.7 | 45.84 | 28.32 | 55.34 | 46.87 | 31.32 | 52.23 |
| 15 | 56hrs | 45.12 | 13.07 | 62.79 | 46.55 | 19.93 | 50.35 | 46.07 | 29.86 | 50.21 |
| 16 | 60hrs | 45.74 | 13.07 | 56.54 | 47.11 | 26.97 | 52.35 | 45.59 | 21.74 | 50.21 |
| 17 | 64hrs | 46.01 | 14.13 | 50.51 | 46.71 | 32.34 | 55.34 | 46.1 | 26.37 | 50.21 |
| 18 | 68hrs | 46.76 | 1.05 | 50.51 | 45.84 | 24.19 | 50.35 | 46.44 | 20.62 | 50.21 |
| 19 | 72hrs | 47.55 | 26.38 | 55.49 | 45.35 | 22.24 | 50.35 | 46.65 | 26.37 | 50.21 |
| 20 | 76hrs | 46.7 | 17 | 59.7 | 45.77 | 23.2 | 50.35 | 46.66 | 26.81 | 50.21 |
| 21 | 80hrs | 46.17 | 33.1 | 56.54 | 46.47 | 21.08 | 50.35 | 45.7 | 27.86 | 50.2 |

🔬 BATCH 381–382–383

1️⃣ RAW MATERIAL COMPOSITION — BIOLOGICAL READING

Structure

- New paddy 53%

- New bagasse 36.2%

- Wheat 10.8%

👉 Verdict
✔ Balanced and proven structure
✔ Wheat gives useful stability
✔ Good formula base for uniform compost

⚠ Still moderately sensitive to airflow because mustard is absent.

2️⃣ LAGOON WATER & SALT LOAD

- EC progression: 1.37 → 3.83 → 2.37 → 1.89

👉 Verdict
✔ Good washing
✔ Final EC is safe
✔ No major salt-stress risk

This is not a lagoon-failure batch.

3️⃣ STAGE-0 PERFORMANCE

- 0B MC: 73.6%

- Reload MC: 72.5%

- Smell: Sweet

- Loading marked deviation Y

- Temperatures stayed around 57–59°C

👉 Interpretation
✔ Moisture good
✔ No souring
✔ Stage-0 is biologically clean

⚠ Slight deviation at loading suggests early non-uniform distribution, but not a major failure.

4️⃣ PHASE-1 (BUNKER) ANALYSIS

Loading

- Time to 68–72°C: 12 vs 5 hrs ❗

- Holding: 30 vs 29 hrs

👉 Major early imbalance.

Reload-1

- Time to peak: 19 vs 15 hrs

- Holding: 22 vs 29 hrs

👉 Moderate imbalance continues.

Reload-2

- Time to peak: 11 vs 12 hrs

- Holding: 23 vs 19 hrs

- Ammonia: Mild

👉 Reload-2 becomes better synchronized.

5️⃣ PHASE-1 VERDICT

✔ Biology strong
✔ Final finish acceptable

❗ Main weakness:

- loading-stage imbalance

- moderate carryover variation into Reload-1

This means compost enters Phase-2 with some early maturity difference already built in.

6️⃣ PHASE-2 ANALYSIS

Tunnel Entry

- MC: 74%

- EC: 1.50

- Fill heights: 2.1 / 1.85 / 1.95 m

👉 Verdict
✔ Entry moisture and EC are good
⚠ Fill-height variation remains significant

Process

- Pasteurization: 11 / 9 / 9 hrs

- Conditioning-2: 91 / 91 / 100 hrs

- Cooling-2:

  - Tunnel 10: 13+0

  - Tunnel 3: 15+0

  - Tunnel 4: 5+16

👉 Interpretation
✔ Pasteurization stable
⚠ Tunnel 4 is slightly more conditioned than the others
⚠ Process uniformity is acceptable, but not ideal

7️⃣ COMPOST-OUT QC

- MC: 66.2–66.8%

- N: 1.89–2.08

- pH: 7.34–7.64

- EC: 1.73 / 1.80 / 2.05

- Spring: Pass

- Color: Dark chocolate

- Actinomycetes: Mild to Strong

- Smell: Earthy

👉 Verdict
✔ Compost-out is usable and mature
✔ Structure held well
⚠ Tunnel 4 shows the highest EC (2.05) and lowest N (1.89)
⚠ Compost quality is good, but not equally uniform across all tunnels

8️⃣ BIOLOGICAL CHARACTER OF THIS BATCH

Strengths

- Good formula

- Good lagoon washing

- Clean Stage-0

- Acceptable compost-out quality

Weaknesses

- Loading-stage imbalance

- Tunnel fill-height variation

- Tunnel 4 slightly more selective / more conditioned than others

9️⃣ EXPECTED GROWING-ROOM BEHAVIOR

| Observation | Cause |
| --- | --- |
| mild patching | early bunker imbalance |
| room-to-room timing difference | tunnel conditioning variation |
| striping under stress | airflow amplifying existing variation |
| slightly different pin density | compost-out non-uniformity |

🔟 AIR VELOCITY SENSITIVITY

👉 Moderate

Not because of salt.
Because:

- paddy is dominant

- tunnel heights differ

- one tunnel finished slightly differently

Airflow will amplify the difference, but airflow is not the primary root cause.

1️⃣1️⃣ FINAL TECHNICAL VERDICT

Batch 381–383 is a biologically good batch with acceptable compost-out, but it is not perfectly uniform.

✔ Salt stress: none
✔ Stage-0: good
✔ Compost-out: good

❗ Main weaknesses:

- loading-stage imbalance

- fill-height variation

- one tunnel slightly over-conditioned / higher EC at compost-out

🧠 ONE-LINE FIELD SUMMARY

381–383 is a good batch, but not a perfectly even one; any later room variation will come mainly from early bunker imbalance and tunnel-to-tunnel process difference, not from salt or Stage-0 failure.