| Master Batch No:384,385,386 |  |  |  |
| --- | --- | --- | --- |
| SECTION–0: MASTER BATCH IDENTIFICATION (ONE TIME) |  |  |  |
| Date of Batch Start | 21/03/26 |  |  |
| No. of Rooms Covered | 3 |  |  |
| Raw Material Basis except minerals | Bagasse new%: 36 | Paddy new%: 53 | Wheat Straw%: 11 |
| Target Compost Qty (t) | 150MT (50MT each room) |  |  |
| Supervisor / In-charge | RAMARAO |  |  |

| Section 1A: RAW MATERIALS COMPOSITION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Raw materials | Age (months) | Dry(wt) Qty (MT) | Fresh (wt) Qty (MT) | PH | Moisture % | N% | Ash% |
| Paddy straw new | 4 | 29.4 | 33.4 | 6.81 | 12 | 0.77 | 12.7 |
| Bagasse new | 4 | 20.1 | 47.65 | 5.60 | 57.8 | 0.46 | 2.2 |
| Wheat straw | 1 | 6.0 | 6.77 | 5.62 | 11.4 | 0.70 | 10.3 |
| Chicken Manure |  | 20.4 | 24.82 | 7.89 | 17.8 | 3.48 | 30.3 |
| Gypsum |  | 4.2 | 5.88 | 3.56 | 28.5 | - | 84.4 |
| AS |  | 0.55 | 0.55 | 5.24 | - | 20.5 | - |
| Total quantity |  | 80.65 | 119.07 |  |  |  |  |

| LAGOON WATER QUALITY TESTING FOR PADDY(NEW) SOAKING |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Before soaking | After Soaking-1 | After Soaking -2 | After Soaking – 3 |
| Water volume(lts) | 150 KL | 150 KL | 150 KL | 150 KL |
| Colour | Light Brown | Brown | Brown | Brown |
| PH | 7.19 | 5.86 | 7.19 | 7.50 |
| EC | 1.23 | 3.12 | 1.64 | 1.4 |
| TDS | 1.29 | 5.00 |  |  |

| 1B. Stage–0 Summary |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Stage | MC % | Temperature (45-55°C) |  |  | Smell(sweet/sour) | Deviations(Y/N) |
|  |  | 0 hrs | 24 hrs | 48 hrs |  |  |
| 0B (Bunker Loading)7 | 72.8 | 40.12 | 57.90 | 56.42 | Sweet | N |
| 0B (Bunker Loading) 8 | 72.8 | 41.01 | 58.10 | 56.01 | Sweet | N |
| 0D R/L (Reloading) | 72 | 49.32 | 58.67 | 52.48 | Sweet | N |
| Unloading |  |  |  |  |  |  |

| ### SECTION–2: PHASE–1 – BUNKER-WISE RECORD |  |  |  |
| --- | --- | --- | --- |
| ### (3 batches loading into 2 bunkers) |  |  |  |
| Loading | BUNKER No: 3 | BUNKER No: 4 | Remarks |
| Compost Qty (MT) | 150 | 150 |  |
| Start Fill Height (m) | 2.9 | 2.9 |  |
| Settled Height @ 4h / 8h | 2.6/2.2 | 2.6/2.2 |  |
| MC % | 74.5 | 74.5 |  |
| EC | 1.73 | 1.73 |  |
| Time to Reach (68–72°C) | 9 | 9 |  |
| Holding time (hrs) | 33 | 34 |  |
| Ammonia smell Observation time(hrs) | 5 | 6 |  |
| Ammonia smell (strong/ mild/ weak) | Strong | Strong |  |
| Spring Test (Pass / Fail) | Pass | Pass |  |
| Reloading -1 | BUNKER No: 5 | BUNKER No: 6 |  |
| Reload Height (m) | 2.9 | 2.9 |  |
| MC % after Reload-1 | 75 | 75 |  |
| EC | 1.64 | 1.64 |  |
| Time to Reach 68–72°C | 11 | 16 |  |
| Holding time (hrs) | 36 | 30 |  |
| Spring Test (Pass / Fail) | Pass | Pass |  |
| Reloading -2 | BUNKER No: 4 | BUNKER No: 1 |  |
| Reload Height (m) | 2.8 | 2.8 |  |
| MC % after Reload-2 | 74.5 | 74.5 |  |
| EC | 1.52 | 1.52 |  |
| Time to Reach 68–72°C | 11 | 18 |  |
| Holding time (hrs) | 21 | 9 |  |
| Ammonia smell (strong/ mild/ weak) | Mild | Mild |  |

| SECTION–3: PHASE–2 – TUNNEL-WISE RECORD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3A. Tunnel Allocation (2 bunkers filled in 3 tunnels) |  |  |  |  |  |
| Reloading Bunker No: | Tunnel No: | Moisture% | EC | Compost Qty (MT) | Fill Height (m) |
| 1&4 | 8 | 74.5 | 1.57 | 70 | 2.1 |
| 1&4 | 9 | 74.5 | 1.57 | 70 | 2.1 |
| 1&4 | 2 | 74.5 | 1.57 | 70 | 1.5 |

| Stage | Temp Range (°C) | Batch No/Tunnel No Time to reach desired T + Time Held (in hrs) |  |  | Min–Max T probes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 384/2 | 385/9 | 386/8 | 384/2 | 385/9 | 386/8 |
| Levelling + Conditioning-1 (18 – 20 hrs) | 46 - 49 | 19+0 | 7+11 | 12+7 | 48.45-50.06 | 48.93-51.37 | 48.45-50.06 |
| Heating-Up (8-12 hrs) | 59 | 20+2 | 7+1 | 11+0 | 58.61-59.60 | 58.59-59.96 | 58.31-60.89 |
| Pasteurization (8 – 10 hrs) | 59 (58-60) | 11 | 9 | 9 | 57.80-58.77 | 59.23-60.00 | 56.38-60.82 |
| Cooling-1 (12 – 14 hrs) | 60-48 | 7+6 | 9+4 | 7+6 | 46.74-47.87 | 45.62-47.55 | 46.64-49.37 |
| Conditioning-2 (75 - 80 hrs) | 48-45 | 105 | 87 | 91 | 45.68-46.38 | 45.59-46.72 | 45.54-48.21 |
| Cooling-2 (8 - 14hrs) | 24 | 3+0 | 5+0 | 9+9 | 24.33-27.18 | 26.01-27.50 | 24.26-25.40 |

| SECTION–4: COMPOST OUT – TUNNEL-WISE QC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tunnel No | Final MC % | N % | pH | EC | Spring | Color (Grey brown/Dark chocolate) | Actinomycetes (Visible-weak/ mild/strong) | Smell (Sour/ Earthy) |
| 9 | 65.5 | 2.03 | 7.39 | 2.03 | Pass | Dark chocolate | Strong | Earthy |
| 8 | 66.7 | 2.0 | 7.42 | 2.00 | Pass | Dark chocolate | Strong | Earthy |
| 2 | 65.8 | 1.94 | 7.43 | 1.94 | Pass | Dark chocolate | Strong | Earthy |

| Sr. No | Hour | 384/2 |  |  | 385/9 |  |  | 386/8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan |
| 1 | 0hrs | 55.83 | 0.3 | 60.97 | 46.18 | 28.04 | 70.49 | 47.35 | 20.02 | 69.56 |
| 2 | 4hrs | 51.71 | 15.34 | 83.82 | 47.6 | 39.85 | 70.49 | 48.81 | 20.02 | 67.34 |
| 3 | 8hrs | 48.37 | 20.52 | 60.97 | 48.04 | 39.85 | 70.49 | 48.45 | 20.02 | 66.55 |
| 4 | 12hrs | 47.33 | 14.58 | 60.96 | 47.87 | 39.85 | 70.49 | 48.48 | 35.05 | 70.6 |
| 5 | 16hrs | 44.74 | 24.42 | 66.13 | 47.29 | 39.85 | 72.51 | 46.52 | 39.96 | 68.58 |
| 6 | 20hrs | 44.49 | 0.3 | 40.32 | 46.67 | 39.85 | 69.52 | 45.26 | 39.96 | 62.57 |
| 7 | 24hrs | 47.38 | 5.15 | 50.56 | 46.29 | 39.85 | 63.42 | 46.21 | 39.96 | 50.12 |
| 8 | 28hrs | 48.71 | 19.43 | 50.69 | 46.44 | 39.85 | 56.32 | 46.7 | 39.96 | 50.13 |
| 9 | 32hrs | 47.36 | 19.42 | 54.56 | 46.64 | 39.85 | 56.32 | 46.87 | 39.96 | 50.12 |
| 10 | 36hrs | 46.5 | 19.42 | 50.75 | fault | 24.95 | 11.36 | 46.34 | 30.06 | 50.12 |
| 11 | 40hrs | 45.94 | 19.42 | 50.7 | 46.58 | 39.85 | 56.33 | 45.71 | 39.96 | 50.12 |
| 12 | 44hrs | 45.6 | 1.26 | 50.67 | 46.25 | 39.85 | 51.13 | 44.73 | 39.96 | 50.12 |
| 13 | 48hrs | 46.69 | 10.21 | 50.76 | 46.31 | 39.85 | 50.1 | 45.62 | 7 | 50.12 |
| 14 | 52hrs | 46.6 | 16.42 | 50.76 | 46.1 | 29.96 | 50.1 | 45.82 | 30.49 | 50.13 |
| 15 | 56hrs | 45.72 | 19.45 | 50.76 | 46.75 | 23.98 | 50.1 | 46.07 | 30.76 | 50.13 |
| 16 | 60hrs | 44.64 | 0.32 | 50.64 | 47.16 | 39.85 | 55.22 | 46.07 | 35.05 | 50.12 |
| 17 | 64hrs | 45.77 | 0.31 | 50.58 | 46.46 | 39.85 | 55.42 | 45.51 | 39.96 | 50.12 |
| 18 | 68hrs | 46.42 | 0.31 | 50.56 | 46.34 | 39.85 | 50.1 | 45.76 | 20.02 | 50.13 |
| 19 | 72hrs | 46.93 | 0.3 | 50.6 | 46.44 | 39.85 | 50.1 | 46.14 | 20.02 | 50.13 |
| 20 | 76hrs | 47.17 | 0.31 | 55.73 | 46.32 | 39.85 | 50.1 | 46.29 | 20.02 | 50.12 |
| 21 | 80hrs | 44.28 | 5.4 | 56.71 | 46.09 | 39.85 | 50.1 | 46.37 | 20.02 | 50.12 |

🔬 BATCH 384–385–386

TECHNICAL COMPOST & BIOLOGY ANALYSIS

1️⃣ RAW MATERIAL COMPOSITION — BIOLOGICAL READING

Structure

- New paddy 53%

- New bagasse 36%

- Wheat 11%

👉 Verdict
✔ Balanced and biologically stable structure
✔ Wheat gives useful fiber support
✔ Good formula base for uniform composting

⚠ Still moderately sensitive to airflow because no mustard buffer is present.

2️⃣ LAGOON WATER & SALT LOAD

- EC progression: 1.23 → 3.12 → 1.64 → 1.40

👉 Verdict
✔ Very good washing
✔ Low final salt load
✔ This is not a salt-stress batch

So later issues should not be attributed to lagoon salinity.

3️⃣ STAGE-0 PERFORMANCE

- 0B MC: 72.8%

- Reload MC: 72.0%

- Smell: Sweet

- Temperatures stayed around 56–58°C

👉 Interpretation
✔ Good moisture
✔ No souring
✔ Stage-0 is clean and controlled

This is not a Stage-0 failure batch.

4️⃣ PHASE-1 (BUNKER) ANALYSIS

Loading

- Time to 68–72°C: 9 vs 9 hrs

- Holding: 33 vs 34 hrs

👉 Very good synchronization.

Reload-1

- Time to peak: 11 vs 16 hrs

- Holding: 36 vs 30 hrs

👉 Moderate imbalance begins.

Reload-2

- Time to peak: 11 vs 18 hrs

- Holding: 21 vs 9 hrs

- Ammonia: Mild

👉 Major imbalance

5️⃣ PHASE-1 VERDICT

✔ Biology is active
✔ Loading stage was strong and even
❗ Main weakness = Reload-2 non-uniformity

This means:

- compost maturity diverged late in Phase-1

- different compost strength likely entered tunnels

6️⃣ PHASE-2 ANALYSIS

Tunnel Entry

- MC: 74.5%

- EC: 1.57

- Fill heights: 2.1 / 2.1 / 1.5 m

👉 Verdict
✔ Entry moisture and EC are very good
❗ Fill height variation is too high

2.1 m vs 1.5 m is a major process difference.

Process

- Pasteurization: 11 / 9 / 9 hrs

- Conditioning-2: 105 / 87 / 91 hrs

- Cooling-2: 3+0 / 5+0 / 9+9

👉 Interpretation
✔ Pasteurization acceptable
❗ Tunnel 2 is clearly over-conditioned at 105 hrs
❗ Tunnel-to-tunnel process spread is high

7️⃣ COMPOST-OUT QC

- Final MC: 65.5 / 66.7 / 65.8%

- pH: 7.39–7.43

- EC: 1.94 / 2.03 / 2.00

- Spring: Pass

- Color: Dark chocolate

- Actinomycetes: Strong

- Smell: Earthy

👉 Verdict
✔ Compost-out is usable and mature
✔ Biological finish is good
⚠ EC is slightly higher than your cleaner batches
⚠ Final quality is acceptable, but not equally uniform

8️⃣ BIOLOGICAL CHARACTER OF THIS BATCH

Strengths

- Good formula

- Good lagoon washing

- Clean Stage-0

- Strong compost-out biology

Weaknesses

- Late Phase-1 divergence

- Very uneven tunnel fill height

- One tunnel over-conditioned

- Slightly higher final EC than best batches

9️⃣ EXPECTED GROWING-ROOM BEHAVIOR

| Observation | Cause |
| --- | --- |
| room-to-room timing variation | Reload-2 mismatch |
| patch tendency | late bunker non-uniformity |
| uneven pin response | tunnel fill-height and conditioning spread |
| striping under stress | airflow amplifying existing variation |

🔟 AIR VELOCITY SENSITIVITY

👉 Moderate to high

Not because of salt.
Because:

- paddy is dominant

- tunnel heights differ sharply

- one tunnel is over-conditioned

- compost-out is good but not equally even

Airflow will amplify the process variation already built into the batch.

1️⃣1️⃣ FINAL TECHNICAL VERDICT

Batch 384–386 is a biologically good batch, but it is process-disturbed in late Phase-1 and Phase-2.

✔ Salt stress: none
✔ Stage-0: good
✔ Compost-out: acceptable to good

❗ Main weaknesses:

- Reload-2 imbalance

- 2.1 m vs 1.5 m tunnel fill variation

- Tunnel 2 over-conditioning

🧠 ONE-LINE FIELD SUMMARY

384–386 is not a chemistry failure batch; its real problem is late process non-uniformity from Reload-2 mismatch and highly uneven tunnel filling.