| Master Batch No: 387,388,389,390 |  |  |  |
| --- | --- | --- | --- |
| SECTION–0: MASTER BATCH IDENTIFICATION (ONE TIME) |  |  |  |
| Date of Batch Start | 23/03/26 |  |  |
| No. of Rooms Covered | 4 |  |  |
| Raw Material Basis except minerals | Bagasse new%: 38 | Paddy new%: 53 | Wheat Straw%: 9 |
| Target Compost Qty (t) | 200MT (50MT each room) |  |  |
| Supervisor / In-charge | RAMARAO |  |  |

| Section 1A: RAW MATERIALS COMPOSITION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Raw materials | Age (months) | Dry(wt) Qty (MT) | Fresh (wt) Qty (MT) | PH | Moisture % | N% | Ash% |
| Paddy straw new | 4 | 40.0 | 46.4 | 6.62 | 13.8 | 0.90 | 15.4 |
| Bagasse new | 4 | 29.2 | 65.8 | 5.63 | 55.4 | 0.42 | 1.50 |
| Wheat straw | 1 | 6.8 | 7.7 | 5.74 | 11.4 | 0.70 | 9.9 |
| Chicken Manure |  | 25.6 | 30.9 | 7.89 | 17.6 | 3.48 | 30.3 |
| Gypsum |  | 5.2 | 7.24 | 3.50 | 28.2 | - | 84.4 |
| AS |  | 0.90 | 0.90 | 5.32 | - | 20.5 | - |
| Total quantity |  | 107.7 | 158.94 |  |  |  |  |

| LAGOON WATER QUALITY TESTING FOR PADDY(NEW) SOAKING |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Before soaking | After Soaking-1 | After Soaking -2 | After Soaking – 3 |
| Water volume(lts) | 150 KL | 150 KL | 150 KL | 150 KL |
| Colour | Light Brown | Brown | Brown | Brown |
| PH | 6.87 | 4.28 | 4.82 | 6.13 |
| EC | 0.784 | 4.83 | 2.99 | 1.75 |
| TDS | 1.45 | 8.80 | 5.50 | 3.40 |

| 1B. Stage–0 Summary |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Stage | MC % | Temperature (45-55°C) |  |  | Smell(sweet/sour) | Deviations(Y/N) |
|  |  | 0 hrs | 24 hrs | 48 hrs |  |  |
| 0A Pre-wetting |  |  |  |  |  |  |
| 0B (Bunker Loading)1 | 71.9 | 40.85 | 57.11 | 57.62 | Sweet | N |
| 0D R/L (Reloading) 2 | 71.8 | 46.33 | 61.95 | 57.85 | Sweet | Y |
| Unloading |  |  |  |  |  |  |

| ### SECTION–2: PHASE–1 – BUNKER-WISE RECORD |  |  |  |
| --- | --- | --- | --- |
| ### (3 batches loading into 2 bunkers) |  |  |  |
| Loading | BUNKER No: 7 | BUNKER No: 8 | BUNKER No: 3 |
| Compost Qty (MT) | 140 | 140 | 140 |
| Start Fill Height (m) | 2.9 | 2.9 | 2.9 |
| Settled Height @ 4h / 8h | 2.7/2.2 | 2.7/2.2 | 2.7/2.1 |
| MC % | 76 | 76 | 76 |
| EC | 1.64 | 1.64 | 1.64 |
| Time to Reach (68–72°C) | 9 | 9 | 19 |
| Holding time (hrs) | 27 | 25 | 24 |
| Ammonia smell Observation time(hrs) | 5 | 5 | 8 |
| Ammonia smell (strong/ mild/ weak) | Strong | Strong | Strong |
| Spring Test (Pass / Fail) | Pass | Pass | Pass |
| Reloading -1 | BUNKER No: 5 | BUNKER No: 6 | BUNKER No: 7 |
| Reload Height (m) | 2.8 | 2.8 | 2.8 |
| MC % after Reload-1 | 75.6 | 75.6 | 75.6 |
| EC | 1.67 | 1.67 | 1.67 |
| Time to Reach 68–72°C | 12 | 13 | 12 |
| Holding time (hrs) | 40 | 34 | 31 |
| Spring Test (Pass / Fail) | Pass | Pass | Pass |
| Reloading -2 | BUNKER No: 8 | BUNKER No:9 | BUNKER No:10 |
| Reload Height (m) | 2.7 | 2.7 | 2.7 |
| MC % after Reload-2 | 75.6 | 75.6 | 75.6 |
| EC | 1.60 | 1.60 | 1.60 |
| Time to Reach 68–72°C | 11 | 20 | 10 |
| Holding time (hrs) | 34 | 22 | 26 |
| Ammonia smell (strong/ mild/ weak) | Mild | Mild | Mild |

| SECTION–3: PHASE–2 – TUNNEL-WISE RECORD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3A. Tunnel Allocation (2 bunkers filled in 3 tunnels) |  |  |  |  |  |
| Reloading Bunker No: | Tunnel No: | Moisture% | EC | Compost Qty (MT) | Fill Height (m) |
| 8,9&10 | 6 | 74.8 | 1.49 | 80 | 1.95 |
| 8,9&10 | 5 | 74.8 | 1.49 | 80 | 1.95 |
| 8,9&10 | 12 | 74.8 | 1.49 | 80 | 2.1 |
| 8,9&10 | 11 | 74.8 | 1.49 | 80 | 1.85 |

| Stage | Temp Range (°C) | Batch No/Tunnel No Time to reach desired T + Time Held (in hrs) |  |  |  | Min–Max T probes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 387/6 | 388/12 | 389/5 | 390/11 | 387/6 | 388/12 | 389/5 | 390/11 |
| Levelling + Conditioning-1 (18 – 20 hrs) | 46 - 49 | 5+15 | 8+11 | 8+10 | 10+9 | 49.48-49.97 | 47.65-51.33 | 48.94-50.69 | 48.53-51.84 |
| Heating-Up (8-12 hrs) | 59 | 15+0 | 11+0 | 13+0 | 24+4 | 58.19-59.13 | 578.34-59.63 | 57.45-58.97 | 57.44-60.28 |
| Pasteurization (8 – 10 hrs) | 59 (58-60) | 9 | 11 | 9 | 9 | 57.91-58.81 | 58.02-59.77 | 58.41-59.82 | 57.07-59.50 |
| Cooling-1 (12 – 14 hrs) | 60-48 | 7+6 | 8+5 | 10+3 | 8+2 | 46.16-47.86 | 46.05-49.21 | 46.14-49.36 | 41.75-44.02 |
| Conditioning-2 (75 - 80 hrs) | 48-45 | 92 | 91 | 102 | 92 | 45.59-46.98 | 45.12-46.61 | 44.95-47.72 | 45.05-47.28 |
| Cooling-2 (8 - 14hrs) | 24 | 10+0 | 13+0 | 13+12 | 14+6 | 26.14-26.63 | 23.85-26.43 | 22.83-26.95 | 23.92-25.86 |

| SECTION–4: COMPOST OUT – TUNNEL-WISE QC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tunnel No | Final MC % | N % | pH | EC | Spring | Color (Grey brown/Dark chocolate) | Actinomycetes (Visible-weak/ mild/strong) | Smell (Sour/ Earthy) |
| 6 | 68.4 | 2.09 | 7.32 | 1.86 | Pass | Dark chocolate | Mild | Earthy |
| 12 | 68 | 2.14 | 7.40 | 1.90 | Pass | Dark chocolate | Mild | Earthy |
| 5 | 67.8 | 2.10 | 7.44 | 1.89 | Pass | Dark chocolate | Strong | Earthy |
| 11 | 67.2 | 2.09 | 7.23 | 1.82 | Pass | Dark chocolate | Strong | Earthy |

| Sr. No | Hour | 387/6 |  |  | 388/12 |  |  | 389/5 |  |  | 390/11 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan | Avg ℃ | OA | Fan |
| 1 | 0hrs | 47.41 | 14.81 | 67.41 | 47.54 | 39.29 | 61.41 | 47.85 | 21.13 | 65.52 | 45.8 | 0.08 | 29.83 |
| 2 | 4hrs | 47.7 | 26.33 | 61.04 | 48 | 43.15 | 54.04 | 47.88 | 20.17 | 60.35 | 50.37 | 45.08 | 51.22 |
| 3 | 8hrs | 47.96 | 31.96 | 56.85 | 48.42 | 43.18 | 56.85 | 48.21 | 31.26 | 57.3 | 46.26 | 9.02 | 50.24 |
| 4 | 12hrs | 46.29 | 27.6 | 52.84 | 48.06 | 43.26 | 62.84 | 47.07 | 29.2 | 53.29 | 48.44 | 42 | 52.17 |
| 5 | 16hrs | 46.38 | 23.16 | 50.59 | 46.24 | 43.89 | 50.59 | 47.33 | 50.26 | 50.13 | 45.9 | 33.01 | 54.23 |
| 6 | 20hrs | 46.61 | 26.21 | 50.65 | 46.06 | 35.33 | 50.65 | 45.64 | 0.25 | 50.12 | 44.34 | 0.09 | 40.04 |
| 7 | 24hrs | 46.45 | 21.98 | 50.62 | 46.7 | 42.43 | 50.62 | 46.72 | 30.16 | 50.12 | 47.62 | 30.06 | 40.03 |
| 8 | 28hrs | 46.58 | 23.94 | 50.59 | 46.9 | 42.49 | 50.59 | 46.7 | 17.17 | 50.12 | 49.56 | 30.06 | 39.91 |
| 9 | 32hrs | 47.14 | 30.7 | 54.84 | 47.04 | 42.04 | 54.84 | 47.25 | 29.33 | 52.25 | 48.89 | 40.02 | 60.39 |
| 10 | 36hrs | 45.59 | 30.15 | 52.84 | 46.38 | 43.71 | 52.84 | 46.79 | 40.17 | 56.35 | 42.04 | 0.09 | 50.24 |
| 11 | 40hrs | 45.9 | 13.16 | 50.59 | 46 | 43.71 | 50.59 | 44.44 | 0.25 | 52.3 | 45 | 0.09 | 50.25 |
| 12 | 44hrs | 45.98 | 25.11 | 50.66 | 45.73 | 43.69 | 50.66 | 46.05 | 19.22 | 50.13 | 46.54 | 36.08 | 50.25 |
| 13 | 48hrs | 46.33 | 19.69 | 50.59 | 46.26 | 37.23 | 50.59 | 46.29 | 22.22 | 50.13 | 46.04 | 14.06 | 50.24 |
| 14 | 52hrs | 46.79 | 27.61 | 50.79 | 46.3 | 42.42 | 50.79 | 46.37 | 16.2 | 50.13 | 46.55 | 30.05 | 50.25 |
| 15 | 56hrs | 46.63 | 27.73 | 50.82 | 46.12 | 42.46 | 50.82 | 46.6 | 20.18 | 50.12 | 45.6 | 40.02 | 50.24 |
| 16 | 60hrs | 45.39 | 21.97 | 50.82 | 45.55 | 42.05 | 50.82 | 46.06 | 37.16 | 50.13 | 43.71 | 0.09 | 50.24 |
| 17 | 64hrs | 45.84 | 13.61 | 50.58 | 45.03 | 33.46 | 50.58 | 44.85 | 0.25 | 50.14 | 45.47 | 0.09 | 50.24 |
| 18 | 68hrs | 45.06 | 2.63 | 50.59 | 46.33 | 42.27 | 50.59 | 45.96 | 16.97 | 50.13 | 46.13 | 35.09 | 50.24 |
| 19 | 72hrs | 46.02 | 18.99 | 50.59 | 46.42 | 42.27 | 50.59 | 45.91 | 16.2 | 50.13 | 45.9 | 4.1 | 50.24 |
| 20 | 76hrs | 46.38 | 28.66 | 50.82 | 46.5 | 42.04 | 50.82 | 46.21 | 12.25 | 50.12 | 46.3 | 30.06 | 50.25 |
| 21 | 80hrs | 45.33 | 24.44 | 50.82 | 46.6 | 42.04 | 50.82 | 46.67 | 24.13 | 50.12 | 45.52 | 34.85 | 50.24 |

🔬 BATCH 387–388–389–390

FINAL TECHNICAL ANALYSIS

1️⃣ Raw Material & Lagoon

- New paddy 53%

- New bagasse 38%

- Wheat 9%

- Lagoon EC: 0.784 → 4.83 → 2.99 → 1.75

👉 Verdict
✔ Formula is good
✔ Third soaking corrected EC well
✔ Not a salt-stress batch

2️⃣ Stage-0

- MC: 71.9 → 71.8%

- Smell: Sweet

- Reload temp reached 61.95°C

👉 Verdict
✔ No souring
⚠ Slight heat overactivity at reload

3️⃣ Phase-1

Main issue

- Loading peak: 9 / 9 / 19 hrs

- Reload-2 peak: 11 / 20 / 10 hrs

👉 One bunker repeatedly behaved slow.

Interpretation

✔ Biology active
❗ Process not fully synchronized
👉 Compost maturity becomes mixed.

4️⃣ Phase-2

- Tunnel entry MC: 74.8%

- EC: 1.49

- Fill height: 1.85–2.1 m

- Conditioning-2: 92 / 91 / 102 / 92 hrs

👉 Verdict
✔ Entry condition good
⚠ Tunnel 5 over-conditioned at 102 hrs
⚠ Tunnel 11 had unstable cooling/temperature profile

5️⃣ Compost-Out QC

- MC: 67.2–68.4%

- N: 2.09–2.14

- pH: 7.23–7.44

- EC: 1.82–1.90

- Actinomycetes: Mild to Strong

- Smell: Earthy

👉 Verdict
✔ Final compost quality is good
✔ Moisture is excellent
✔ EC safe
⚠ Biological strength not fully uniform across tunnels

6️⃣ Air Velocity Sensitivity

👉 Moderate

Not due to salt.
Main reason:

- one slow bunker carryover

- tunnel conditioning spread

- slight Stage-0 heat stress

Airflow will amplify room-to-room variation.

Final Verdict

Batch 387–390 is a good-quality compost batch, but not a perfectly uniform batch.

✔ Salt stress: no
✔ Compost-out: good
❗ Main risk: slow-bunker carryover + one over-conditioned tunnel

One-line Summary

387–390 should perform well, but any pinning variation will come from process non-uniformity, not formula or salt.