Freshbowl Mushroom Composting Workflow Automation

Proposed Software Scope and Deliverables

The purpose of the software should be to create a complete digital record for every compost batch, starting from monthly planning and raw-material formulation and continuing until the compost is filled into growing rooms and mushrooms are harvested.

Every activity should capture:

- Planned start and completion time

- Actual start and completion time

- Delay or early completion in hours

- Employee and supervisor responsible

- Equipment, bunker, tunnel or room used

- Process measurements and test results

- Photograph of completed work

- Remarks, deviations and corrective action

- Supervisor approval

- Date-and-time audit trail

Overall workflow

Monthly Batch Planning → Raw-Material Formulation → Raw-Material Testing → Required Weighment → Actual Weighment → Composting Activities → Bunker/Tunnel Tracking → Growing-Room Filling → Harvest Data → Reports and Analysis

1. Monthly Planning of Batches

The software should generate the monthly production and composting plan.

Information to capture

- Month and year

- Batch number

- Rooms planned for the batch

- Number of rooms per batch

- Planned compost quantity

- Planned bunker numbers

- Planned tunnel numbers

- Planned growing-room numbers

- Planned dates and times for:

  - Raw-material preparation

  - Soaking

  - Mixing

  - Bunker loading

  - Reloading

  - Tunnel loading

  - Phase-II completion

  - Growing-room filling

- Estimated tunnel-out date

- Estimated harvest dates

- Batch status:

  - Planned

  - In progress

  - Delayed

  - Completed

  - Cancelled

Software output

- Monthly batch calendar

- Bunker utilisation plan

- Tunnel utilisation plan

- Growing-room availability plan

- Conflict warning where the same bunker, tunnel or room is allocated to two batches

- Delayed-batch alert

- Planned versus actual schedule

2. Formulation of Raw Material for Each Batch

The software should calculate the batch formulation based on the planned compost quantity.

Raw materials

- Paddy straw

- Bagasse

- Chicken manure

- Gypsum

- Ammonium sulphate

- Mustard cake or other de-oiled cakes

- Lime or other additives

- Water

Information to capture

- Raw-material name

- Source or supplier

- Old or new material

- Moisture percentage

- Dry weight

- Fresh weight

- Nitrogen percentage

- Carbon percentage

- Ash percentage

- C:N contribution

- Rate per tonne

- Required quantity

- Approved formulation

- Formulation version

Calculations

- Dry weight of each material

- Fresh weight required

- Total dry matter

- Total nitrogen

- Estimated C:N ratio

- Percentage contribution of each material

- Estimated batch cost

- Quantity per room

- Quantity per bunker

The software should retain the original formulation and all later revisions.

3. Testing of Raw Materials

A quality-control module should record laboratory and physical test results before materials are approved for use.

Paddy straw and bagasse tests

- Moisture

- EC

- pH

- Nitrogen

- Carbon

- C:N ratio

- Ash

- Colour

- Smell

- Age of material

- Visible mould

- Contamination

- Storage condition

Chicken manure tests

- Moisture

- Nitrogen

- pH

- EC

- Ash

- Ammonia condition

- Age

- Source farm

- Visual quality

Other materials

- Gypsum moisture and purity

- Ammonium sulphate purity

- Cake moisture and nitrogen

- Water pH and EC

- Lagoon-water pH and EC

Software controls

- Approved

- Conditionally approved

- Rejected

- Retest required

A rejected raw-material lot should not be available for batch formulation without authorised approval.

Attachments

- Laboratory report

- Material photograph

- Supplier certificate

- Sample collection photograph

- Remarks and corrective action

4. Determination of Required Raw-Material Weighment

After formulation approval, the software should generate the material requirement sheet.

Requirement sheet

| Material | Required dry weight | Moisture | Required fresh weight | Source lot | Planned weighment |
| --- | --- | --- | --- | --- | --- |

The software should calculate:

Fresh weight required=Required dry weight1-Moisture fraction

Deliverables

- Batch-wise material requirement

- Room-wise quantity

- Supplier-wise requirement

- Lot-wise allocation

- Planned number of vehicle loads

- Weighment instruction sheet

- Material shortage warning

5. Capturing Actual Weighment of Raw Materials

The actual quantity received and used should be captured from the weighbridge or entered through an authorised screen.

Information to capture

- Batch number

- Material

- Supplier

- Raw-material lot number

- Vehicle number

- Weighment slip number

- Gross weight

- Tare weight

- Net weight

- Moisture percentage

- Calculated dry weight

- Date and time

- Weighbridge operator

- Storage location

- Photograph of vehicle/material

- Weighment-slip photograph

Comparison

- Required fresh weight

- Actual fresh weight

- Difference

- Required dry weight

- Actual dry weight

- Percentage variation

- Excess or shortage

- Supervisor approval

The software should prevent accidental double-entry of the same weighment-slip number.

6. Complete Composting Activity Workflow

This will be the core workflow module.

Each batch should move through predefined composting stages. The stages and timing should be configurable by Freshbowl.

Typical activities

- Paddy-straw showering or soaking

- Bagasse preparation

- Chicken-manure preparation

- Raw-material mixing

- Stage-0 activities

- Stage-1 activities

- Bunker loading

- Bunker holding

- Reload-1

- Reload-2

- PS-1

- PS-2

- PS-3

- Additive mixing

- Final mixing

- Tunnel loading

- Phase-II conditioning

- Heating

- Pasteurisation

- Cooling

- Ammonia removal

- Spawning

- Tunnel unloading

Data for every activity

- Activity name

- Batch number

- Planned start

- Planned completion

- Actual start

- Actual completion

- Duration in hours

- Target duration

- Under-time or over-time

- Responsible employee

- Supervisor

- Equipment used

- Bunker or tunnel number

- Quantity handled

- Moisture

- Temperature

- pH

- EC

- Ammonia

- Water added

- Fan timing

- Observations

- Deviation reason

- Corrective action

- Completion photograph

- Supervisor approval

Workflow controls

An employee should:

- Start the activity in the mobile application.

- Record the required readings.

- Upload a photograph.

- Mark the activity completed.

- Send it to the supervisor for approval.

The next activity should open only after the previous compulsory activity is completed or an authorised override is given.

Alerts

- Activity not started on time

- Activity exceeding standard duration

- Missing test reading

- Missing photograph

- Moisture outside target

- EC outside limit

- Temperature outside limit

- Ammonia not cleared

- Bunker or tunnel conflict

- Supervisor approval pending

7. Capturing Growing-Room Filling

The system should maintain complete traceability from compost batch to growing rooms.

Information to capture

- Compost batch number

- Tunnel number

- Tunnel-out date and time

- Growing-room number

- Filling start and completion time

- Quantity filled

- Compost weight per square metre

- Bed area

- Spawn batch

- Spawn quantity

- Filling-machine details

- Compost temperature

- Compost moisture

- Compost pH

- Compost nitrogen

- Ammonia status

- Filling photograph

- Room-filling completion approval

Traceability

The system should answer:

- Which compost batch entered a particular room?

- Which rooms received compost from a particular batch?

- Which tunnel supplied each room?

- Which raw-material lots were used for that compost?

- Which employees handled the batch?

- What were the process deviations?

8. Capturing Mushroom Harvest Data

Harvest data should be connected to the growing room and original compost batch.

Information to capture

- Harvest date

- Shift

- Growing-room number

- Compost batch

- Flush or break number

- Harvest day

- Grade

- Fresh-market quantity

- Canning quantity

- Reject quantity

- Waste quantity

- Total harvested quantity

- Number of pickers

- Picking hours

- Average output per picker

- Supervisor

- Quality remarks

- Mushroom photograph

Calculations

- Yield percentage

- Kilograms per square metre

- Yield per tonne of compost

- First-break yield

- Second-break yield

- Third-break yield

- Harvest duration

- Picker productivity

- Grade recovery

- Rejection percentage

- Batch-wise profitability

9. Reports and Dashboards

Planning reports

- Monthly batch plan

- Bunker schedule

- Tunnel schedule

- Growing-room schedule

- Planned versus actual calendar

- Resource-conflict report

Raw-material reports

- Batch formulation report

- Raw-material consumption

- Planned versus actual quantity

- Material-lot traceability

- Supplier quality report

- Moisture-adjusted consumption

- Nitrogen and C:N report

- Raw-material cost per batch

Composting reports

- Complete batch history

- Activity start and completion timeline

- Under-time and over-time report

- Delay analysis

- Bunker-wise performance

- Tunnel-wise performance

- Moisture trend

- Temperature trend

- EC and pH trend

- Ammonia-clearance report

- Deviations and corrective actions

- Photograph-based completion report

Growing and harvest reports

- Batch-to-room traceability

- Room-wise yield

- Batch-wise yield

- Flush-wise yield

- Yield per square metre

- Yield per tonne of compost

- Harvest-duration report

- Grade recovery report

- Rejection analysis

- Compost quality versus yield analysis

Management dashboard

The dashboard should show:

- Batches planned this month

- Batches in progress

- Delayed batches

- Current activity of every batch

- Bunkers occupied and available

- Tunnels occupied and available

- Rooms due for filling

- Activities pending approval

- Raw-material shortages

- Quality-control failures

- Current harvest performance

- Batch yield comparison

- Target versus actual cost

Common Software Requirements

Batch identification

Every batch should have a unique batch number and QR code. Scanning the QR code should open the batch record.

Mobile and desktop access

- Mobile application for operators and supervisors

- Desktop dashboard for management

- Tablet-friendly forms for factory use

- Offline capture where network connectivity is weak

- Automatic synchronisation when connectivity returns

Photograph controls

Every photograph should carry:

- Batch number

- Activity

- Employee

- Date and time

- Optional location

- Remarks

The software should not permit an old gallery photograph to be used where a live photograph is compulsory, unless authorised.

User roles

- Administrator

- Monthly planner

- Raw-material department

- Laboratory/QC

- Weighbridge operator

- Composting operator

- Composting supervisor

- Tunnel operator

- Growing-room supervisor

- Harvest supervisor

- Management viewer

Approval levels

- Operator completion

- Supervisor verification

- QC approval

- Production-manager approval

- Management override

Audit trail

The system must record:

- Who entered the data

- Who modified it

- Original value

- Revised value

- Date and time of modification

- Reason for modification

- Approval of modification

Data should not be permanently deleted by normal users.

Suggested Batch Status Flow

| Main status | Sub-status examples |
| --- | --- |
| Planned | Formulation pending, RM allocation pending |
| Raw material | Testing, approved, weighing in progress |
| Composting | Mixing, bunker holding, reload, PS activity |
| Phase II | Heating, pasteurisation, conditioning, cooling |
| Ready for filling | QC approval pending, filling scheduled |
| Growing | Spawn run, casing, pinning, harvesting |
| Completed | Harvest closed, batch report completed |

Suggested Development Phases

Phase 1: Planning and master data

- Batch calendar

- Raw-material masters

- Supplier masters

- Bunker, tunnel and room masters

- Standard process timing

Phase 2: Raw-material and weighment modules

- Formulation

- Laboratory testing

- Material requirement

- Actual weighment

Phase 3: Composting workflow

- Activity start and completion

- Readings

- Photographs

- Alerts

- Approvals

- Timeline analysis

Phase 4: Growing-room and harvest linkage

- Tunnel-to-room tracking

- Room filling

- Harvest capture

- Yield calculation

Phase 5: Reports and management dashboard

- Operational reports

- Quality reports

- Delay analysis

- Yield correlation

- Cost analysis

- Management dashboard

The next practical step is to convert this scope into a formal Software Requirement Specification, including screen-wise fields, calculation formulas, user permissions, approval flow and report formats.